CERTIFICATE OF ANALYSIS

| Manufacture Date: | 2011-12-21 |
| --- | --- |
| Analysis Date: | 2011-12-21 |
| Batch NO.: | 2011122101 |

| Product Name: | D-alpha-Tocopheryl acetate |  |
| --- | --- | --- |
| CAS: | 58-95-7 |  |
| Molecular Weight: | 472.75 |  |
| Structure &Formula:: | C31H52O3 |  |
|  | Inspection Standard | Results |
| Appearance: | Pale yellow viscous oily liquid | conforms |
| Identification | Infrared absorption spectrophotometry abstained by the substance conforms to that obtained by the reference substance | comforms |
| Specific rotation | +24℃ min | +26.5℃ |
| Absorbance | Maximum absorbance(284nm):42-45 | conforms |
|  | Minimum absorbance(254nm):7.0-9.0 |  |
| D-alpha-tocopheryl acetate | 80.88%min 1100IU min | 82.0% 1115.2IU |
| Free tocopherol | 1%max | 0.3% |
| Acid value | 2.0mgKOH/g max | 0.8mg |
| Sulphated ash | 0.10%max | 0.01% |
| Residual solvents | Hexane max290ppm Toluene max890ppm | 10ppm 10ppm |
| Heavy metals | 10ppm max | Conforms |
| As | 2ppm max | Conforms |
| Hg | 1ppm max | Conforms |
| Total plate count | 1000 CFU/g max EP/USP | Conforms |
| Yeast &mold | 100 CFU/g max EP/USP | Conforms |
| E.Coli | Negative EP/USP | Negative EP/USP |
| Salmonella | n.d./10g EP/USP | n.d./10g EP/USP |
| staphylococcus | n.d./ g EP/USP | n.d./ g EP/USP |
| Pseudomonas aeruginosa | n.d./ g EP/USP | n.d./g EP/USP |
| Staphylococcus aureus | n.d./ g EP/USP | n.d./ g EP/USP |
| Bile-tolerant gram-negative bacteria | n.d./ g EP/USP | n.d./ g EP/USP |
| Storage: | Stored in a cool dry place. It should be protected from heat/light, oxygen and moisture. Keep the container tightly closed. |  |